SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:  CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I Bodhanapati Rajesh (192472165) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: B Rajesh

Short Answer Test

1.A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Given

  - File size = 150 MB

  - Bandwidth = 50 Mbps

  - Segment size = 1500 bytes

  - Data Link overhead = 40 bytes/frame  Step 1: Convert file size

150 MB = 150 × 106 = 150,000,000 bytes

Step 2: Number of segments

# = 100,000 segments

Since one segment becomes one frame,

Frames = 100,000

Step 3: Total overhead

Overhead per frame = 40 bytes

100,000 × 40 = 4,000,000 bytes

= 4 MB

Step 4: Transmission time

Total transmitted data

150 + 4 = 154 MB

Convert into bits

154 × 8 = 1232 Mb

Bandwidth

50 Mbps

Time

# = 24.64 seconds

Answer

- Number of segments = 100,000

- Number of frames = 100,000

- Total overhead = 4 MB

- Transmission time ≈ 24.64 seconds

Explanation

Transport Layer

- Divides the file into segments.

- Performs sequencing, acknowledgments, flow control, and error recovery.

Data Link Layer

- Encapsulates segments into frames.

- Detects transmission errors using CRC.

- Ensures reliable communication between adjacent devices.

2.A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions.

Explain how flow control improves communication efficiency.

Given

- Sliding window size = 6

- Total frames = 48

- Frame size = 1024 bytes

Step 1: Number of windows

= 8

Windows required = 8

Step 2: Retransmissions

One frame lost in every window

8 × 1 = 8

Retransmissions = 8

Answer

- Windows required = 8

- Retransmitted frames = 8

Flow Control

Flow control prevents the sender from overwhelming the receiver. It regulates the transmission rate, minimizes packet loss, avoids congestion, improves throughput, and ensures efficient communication.

3.A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Given

- IP packet = 12,000 bytes

- MTU = 1500 bytes

- IP header = 20 bytes

Step 1: Payload per fragment

1500 − 20 = 1480 bytes

Step 2: Number of fragments

= 8.1

Hence,

Fragments = 9

Step 3: Header overhead

Each fragment has a 20-byte header.

9 × 20 = 180 bytes

Answer

- Number of fragments = 9

- Total header overhead = 180 bytes

Network Layer Explanation

The Network Layer performs fragmentation when packets exceed the MTU. Each fragment contains its own IP header with identification, fragment offset, and morefragments flag. At the destination, all fragments are reassembled into the original packet, ensuring successful delivery across networks.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions.

Explain how flow control improves communication efficiency.

Given

  - Sliding window size = 6 frames

  - Total frames = 48

  - Frame size = 1024 bytes

Step 1: Number of transmission windows

= 8

Transmission windows = 8

Step 2: Retransmissions

One frame is lost in every window.

8 × 1 = 8

Retransmissions = 8 frames

Final Answer

  - Transmission windows = 8

  - Retransmissions = 8 frames

Flow Control Explanation

Flow control regulates the sender's transmission speed according to the receiver's capacity. It prevents buffer overflow, minimizes congestion, reduces retransmissions, improves throughput, and ensures efficient and reliable communication.

5.A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Given

  - Video file = 600 MB

  - Checkpoint after every 60 MB

  - Failure after 390 MB

Step 1: Number of checkpoints before failure

= 6.5

Completed checkpoints:

60, 120, 180, 240, 300, 360

Checkpoints created = 6

Step 2: Data to retransmit Failure occurs at 390 MB. Last checkpoint = 360 MB Retransmission:

390 − 360 = 30 MB

Final Answer

  - Checkpoints created = 6

  - Data retransmitted = 30 MB

Session Layer Explanation

The Session Layer establishes, maintains, and terminates communication sessions. Synchronization checkpoints allow interrupted transmissions to resume from the last checkpoint instead of restarting from the beginning, saving time and bandwidth.

6.Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Given

- Original file = 900 MB

- Compression = 40%

- Encryption overhead = 5%

Step 1: Compressed file size

40% reduction means 60% remains.

900 × 0.60 = 540 MB

Step 2: Encryption overhead

540 × 0.05 = 27 MB

Encrypted size

540 + 27 = 567 MB

Step 3: Percentage reduction

Reduction

900 − 567 = 333 MB

Percentage

× 100 = 37%

Final Answer

- Compressed file size = 540 MB

- Encrypted file size = 567 MB

- Total reduction = 37%

Presentation Layer Explanation

The Presentation Layer performs data compression, encryption, decryption, and format conversion. Compression reduces transmission time and bandwidth usage, while encryption protects data confidentiality and integrity during transmission.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Given

- Data = 3 GB

- Throughput = 120 Mbps

- Request size = 3 MB

Step 1: Convert data

3 GB = 3000 MB

Step 2: Number of requests

= 1000

Requests = 1000

Step 3: Transmission time

Convert data into megabits

3000 × 8 = 24000 Mb

Time

# = 200 seconds

Final Answer

- Number of requests = 1000

- Transmission time = 200 seconds

Application Layer Explanation

The Application Layer provides services such as FTP, HTTP, SMTP, and DNS. It enables file transfer, user authentication, error reporting, and communication between network applications and users.

8.A packet experiences the following processing delays at different OSI layers:

Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Given

| Layer | Delay |
| --- | --- |
| Application | 4 ms |
| Transport | 3 ms |
| Network | 5 ms |
| Data Link | 2 ms |
| Physical | 1 ms |
| Propagation | 18 ms |
| Transmission | 12 ms |

Total Delay

4 + 3 + 5 + 2 + 1 + 18 + 12

= 45 ms

Final Answer

End-to-end delay = 45 ms

Layer Contributions

- Application Layer: Provides network services to applications.

- Transport Layer: Ensures reliable delivery and flow control.

- Network Layer: Handles routing and logical addressing.

- Data Link Layer: Performs framing and error detection.

- Physical Layer: Transmits bits over the communication medium.

- Propagation Delay: Time for the signal to travel through the medium.

- Transmission Delay: Time required to place all bits onto the transmission medium.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers.  Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Given

- Useful data = 80 MB

- Frame size = 1600 bytes

- Overhead = 80 bytes/frame  Step 1: Useful payload

1600 − 80 = 1520 bytes

Step 2: Number of frames Useful data

80 MB = 80,000,000 bytes

# = 52,632 frames (approx.)

Step 3: Total overhead

52,632 × 80 = 4,210,560 bytes

≈ 4.21 MB

Step 4: Transmission Efficiency

× 100 = 95%

Final Answer

- Frames required ≈ 52,632

- Total overhead ≈ 4.21 MB

- Transmission efficiency = 95%

Explanation

Protocol overhead consumes additional bandwidth because headers and trailers are transmitted along with user data. Excessive overhead reduces efficiency, while optimized protocols improve network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoT enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and  reliable communication.

Given

- Original file = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Frame overhead = 60 bytes

Step 1: Compressed size

70% remains after compression.

240 × 0.70 = 168 MB

Step 2: Convert to bytes

168,000,000 bytes

Step 3: Number of segments

= 120,000

Segments = 120,000

Step 4: Protocol overhead

120,000 × 60 = 7,200,000 bytes

= 7.2 MB

Step 5: Final transmitted data

168 + 7.2 = 175.2 MB

Final Answer

- Compressed file size = 168 MB

- Segments transmitted = 120,000

- Protocol overhead = 7.2 MB

- Total data transmitted = 175.2 MB

Explanation

The Presentation Layer compresses the data and provides encryption if required. The Transport Layer divides the data into segments for reliable delivery. The Data Link Layer encapsulates each segment into frames by adding headers and trailers for error detection. Finally, the Physical Layer transmits the complete framed data across the network. Together, these OSI layers ensure efficient, secure, and reliable communication.